IEEE 754 Subcircuits Description:

SUB1 — Special Case Detector

What This Subcircuit Does

This is the entry checkpoint of the entire circuit. Before any addition or subtraction happens, this subcircuit examines both 16-bit inputs A and B and detects whether either number is a special IEEE 754 value.

It asks 4 questions about A and 4 questions about B:

- Is it Zero? (exponent=00000 AND mantissa=0000000000)

- Is it Denormal? (exponent=00000 AND mantissa≠0)

- Is it Infinity? (exponent=11111 AND mantissa=0000000000)

- Is it NaN? (exponent=11111 AND mantissa≠0)

It outputs 8 one-bit flags answering these questions. If a flag = 1 that condition is true. If flag = 0 that condition is false.

Components and What Each One Does

16-to-3 Splitter (used 2 times — once for A, once for B)

Simple explanation: A 16-bit number coming in is like a 16-lane highway. This splitter separates the highway into 3 smaller roads:

- Road 1 → carries bits 0 to 9 (the mantissa, 10 bits)

- Road 2 → carries bits 10 to 14 (the exponent, 5 bits)

- Road 3 → carries bit 15 (the sign, 1 bit)

Without this splitter we cannot examine individual parts of the number.

5-to-5 Splitter (used 2 times — once for A exponent, once for B exponent)

Simple explanation: The exponent comes out as a 5-bit group from the previous splitter. But to check if ALL 5 bits are 0 or ALL 5 bits are 1, we need each bit on its own separate wire. This splitter takes the 5-bit exponent bundle and gives each bit its own individual wire. Think of it like opening a bundle of 5 wires and spreading them apart so each wire is separate.

10-to-10 Splitter (used 2 times — once for A mantissa, once for B mantissa)

Simple explanation: Same idea as the 5-to-5 splitter but for the 10-bit mantissa. It takes the 10-bit mantissa bundle and separates all 10 bits onto individual wires so each bit can be checked individually.

NOR Gate with 5 inputs (used 2 times — once for A exponent, once for B exponent)

Simple explanation: This gate checks if ALL 5 exponent bits are zero. It takes all 5 individual exponent bits as inputs. It outputs 1 ONLY when every single input is 0. If even one input is 1, it outputs 0.

Think of it like a supervisor checking 5 workers. He raises his hand (output=1) only when ALL 5 workers are sitting down (all bits = 0). If any worker is standing (any bit = 1) he keeps his hand down (output=0).

This detects the condition exponent = 00000.

AND Gate with 5 inputs (used 2 times — once for A exponent, once for B exponent)

Simple explanation: This gate checks if ALL 5 exponent bits are one. It outputs 1 ONLY when every single input is 1. If even one input is 0, it outputs 0.

Same supervisor analogy — this supervisor raises his hand only when ALL 5 workers are standing (all bits = 1).

This detects the condition exponent = 11111.

NOR Gate with 10 inputs (used 2 times — once for A mantissa, once for B mantissa)

Simple explanation: Same as the 5-input NOR gate but checks 10 mantissa bits instead of 5. It outputs 1 only when ALL 10 mantissa bits are zero. This detects mantissa = 0000000000.

NOT Gate (used 2 times — once for A, once for B)

Simple explanation: This is the simplest gate. It flips a 1 to 0 and a 0 to 1. We use it to create the signal "mantissa is NOT zero" by flipping the output of the 10-input NOR gate. When NOR says mantissa IS zero (output=1), NOT flips it to 0. When NOR says mantissa is NOT zero (output=0), NOT flips it to 1.

AND Gate with 2 inputs (used 8 times — one for each output flag)

Simple explanation: Each of these AND gates combines two conditions to produce one final flag. For example:

- A_is_zero = (exponent is all zeros) AND (mantissa is all zeros)

- A_is_infinity = (exponent is all ones) AND (mantissa is all zeros)

- A_is_nan = (exponent is all ones) AND (mantissa is NOT all zeros)

- A_is_denormal = (exponent is all zeros) AND (mantissa is NOT all zeros)

Each AND gate needs both its inputs to be 1 to output 1. If either input is 0 the output is 0.

SUB2 — Special Case Result Generator

What This Subcircuit Does

This subcircuit receives the 8 flags from SUB1 and decides what the final answer should be when a special case is detected. It follows a strict priority order:

- NaN first — if either input is NaN the answer is NaN

- Infinity second — if either input is infinity handle accordingly

- Zero third — if either input is zero return the other input

- Normal — if none of the above, set is_special=0 and let normal arithmetic handle it

It outputs two things:

- is_special (1 bit) — tells the rest of the circuit whether a special case was found

- special_result (16 bits) — the correct answer if a special case was found

Components and What Each One Does

OR Gate for any_nan (2 inputs, 1 bit)

Simple explanation: Checks if EITHER A is NaN OR B is NaN. An OR gate outputs 1 if at least one input is 1. So if A_is_nan=1 OR B_is_nan=1, this gate outputs 1 meaning "at least one input is NaN."

Splitter to extract sign of A (16-to-2, once for A, once for B)

Simple explanation: We need just the sign bit (bit 15) of A and B separately to check if infinity minus infinity equals NaN. This splitter separates bit 15 from the rest of the 16 bits.

XOR Gate for effective sign of B (1 bit, 2 inputs)

Simple explanation: When SUB=1 (subtraction) the sign of B gets flipped. An XOR gate does this perfectly:

- 0 XOR 0 = 0 (positive B, adding → stays positive)

- 0 XOR 1 = 1 (positive B, subtracting → becomes negative)

- 1 XOR 0 = 1 (negative B, adding → stays negative)

- 1 XOR 1 = 0 (negative B, subtracting → becomes positive)

This gives us the effective sign of B after considering the operation.

XOR Gate for signs differ detection (1 bit, 2 inputs)

Simple explanation: Checks if sign of A is different from effective sign of B. XOR outputs 1 when inputs are different and 0 when they are the same. This is used to detect infinity minus infinity — which happens when both are infinity but have opposite signs after applying the SUB operation.

AND Gate for inf minus inf NaN (3 inputs, 1 bit)

Simple explanation: Combines three conditions:

- A is infinity

- B is infinity

- Their signs differ (after applying SUB)

Only when ALL THREE are true do we have the infinity minus infinity situation which gives NaN.

OR Gate for nan_case (2 inputs, 1 bit)

Simple explanation: Combines the two NaN-producing conditions:

- Any input is directly NaN

- Infinity minus infinity situation

If either is true the overall result must be NaN.

OR Gate for any_inf (2 inputs, 1 bit)

Simple explanation: Checks if either A or B is infinity. Used to determine if the result should be infinity.

OR Gate for any_zero (2 inputs, 1 bit)

Simple explanation: Checks if either A or B is zero. When one input is zero the result is simply the other input.

OR Gate for is_special (3 inputs, 1 bit)

Simple explanation: This is the master flag. It combines nan_case, any_inf, and any_zero. If ANY of these three conditions is true then is_special=1 meaning normal arithmetic should be bypassed. If all three are 0 then is_special=0 meaning proceed with normal calculation.

West-facing Splitter joiner for effective_B (16 bit)

Simple explanation: This works in reverse compared to normal splitters. Instead of splitting a wide wire into narrow ones, it joins narrow wires into one wide wire. It takes the lower 15 bits of B unchanged and combines them with the possibly-flipped sign bit to create effective_B — which is B with its sign adjusted for subtraction.

Priority Encoder (Select Bits=2, highest priority = in3)

Simple explanation: Think of this as a judge deciding which case takes priority. It has 4 inputs and outputs a 2-bit number saying which input has the highest priority and is active:

- in3 = nan_case (highest priority, outputs 11)

- in2 = any_inf (second priority, outputs 10)

- in1 = A_is_zero (third, outputs 01)

- in0 = B_is_zero (lowest, outputs 00)

2-to-1 MUX for infinity result selection (16 bit)

Simple explanation: When the infinity case fires, we need to decide: output A or output effective_B? This MUX makes that choice. If A_is_infinity=1 it outputs A. Otherwise it outputs effective_B (which would be B with sign flipped if subtracting).

4-to-1 MUX for special_result (16 bit)

Simple explanation: This is the final selector for what the special result should be. Based on the priority encoder output it picks one of four 16-bit values:

- sel=11 → NaN constant (0111111000000000)

- sel=10 → infinity result from inf_mux

- sel=01 → effective_B (when A is zero)

- sel=00 → A (when B is zero)

Constant NaN (16 bit, value = 0x7E00)

Simple explanation: This is a fixed value hardwired into the circuit representing the standard quiet NaN = 0111111000000000. Whenever a NaN result is needed this constant provides it.

SUB3 — Sign Controller

What This Subcircuit Does

This is the simplest subcircuit in the entire project. It has one job: when we are subtracting (SUB=1) flip the sign of B to turn it into negative B. When we are adding (SUB=0) leave the sign of B unchanged.

This works because: A − B = A + (−B). By flipping B's sign we convert subtraction into addition, allowing the same addition hardware to handle both operations.

Components and What Each One Does

XOR Gate (1 bit, 2 inputs)

Simple explanation: XOR stands for Exclusive OR. It outputs 1 when its two inputs are DIFFERENT and 0 when they are the SAME.

Here is why this perfectly handles sign flipping:

| sign_B | SUB | XOR output | Meaning |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Positive B, adding → stays positive |
| 0 | 1 | 1 | Positive B, subtracting → becomes negative |
| 1 | 0 | 1 | Negative B, adding → stays negative |
| 1 | 1 | 0 | Negative B, subtracting → becomes positive |

When SUB=0 the XOR just passes sign_B through unchanged. When SUB=1 the XOR flips sign_B. This is exactly what we need.

SUB4 — Bit Extractor

What This Subcircuit Does

This subcircuit takes both 16-bit inputs and separates each one into its three IEEE 754 fields: sign, exponent and mantissa. It also restores the hidden bit.

The hidden bit is a feature of IEEE 754 where normal numbers always have a 1 before the decimal point (like 1.something × 2^power) but this leading 1 is not stored in the 16-bit representation to save space. This subcircuit adds it back to make the mantissa 11 bits wide for arithmetic.

For denormal numbers the hidden bit is 0 instead of 1.

Components and What Each One Does

16-to-3 Splitter (used 2 times)

Simple explanation: Same as in SUB1. Splits the 16-bit input into sign (1 bit), exponent (5 bits) and mantissa (10 bits). Like opening a sealed envelope and separating its contents into three piles.

2-to-1 MUX for hidden bit selection (1 bit, used 2 times)

Simple explanation: This MUX decides whether the hidden bit should be 1 (normal number) or 0 (denormal number).

- When is_denormal=0 → selects input 0 → hidden bit = 1

- When is_denormal=1 → selects input 1 → hidden bit = 0

Think of it like a switch: normal numbers get a 1 prepended, denormal numbers get a 0 prepended.

Constant 1 (1 bit, value = 1)

Simple explanation: A hardwired signal always outputting 1. This represents the hidden leading bit for normal numbers. It feeds into input 0 of the hidden bit MUX.

Constant 0 (1 bit, value = 0)

Simple explanation: A hardwired signal always outputting 0. This represents the hidden leading bit for denormal numbers. It feeds into input 1 of the hidden bit MUX.

West-facing Splitter joiner for 11-bit mantissa (used 2 times)

Simple explanation: This joins the hidden bit (1 bit) and the stored mantissa (10 bits) into a combined 11-bit mantissa. The hidden bit goes into bit position 10 (the most significant position) and the stored mantissa fills positions 9 to 0. Like adding a new carriage to the front of a train.

SUB5 — Exponent Comparator

What This Subcircuit Does

Before adding two mantissas they must represent the same power of 2 — their decimal points must be aligned. This subcircuit compares the two 5-bit exponents to find which is larger, calculates the difference between them (the shift amount), and determines which number was smaller (swap flag).

For example adding 4.0 (exponent=17) and 1.0 (exponent=15): the difference is 2 so the smaller number's mantissa must be shifted right by 2 before adding.

Components and What Each One Does

Comparator (5 bit, unsigned)

Simple explanation: Like a judge comparing two numbers. It has two 5-bit inputs (exp_A and exp_B) and three 1-bit outputs:

- A > B output → 1 when exp_A is bigger

- A = B output → 1 when both are equal

- A < B output → 1 when exp_B is bigger

We use the A < B output directly as swap_flag. When swap_flag=1 it means B has the larger exponent so we need to treat B as the reference.

Subtractor for exp_A minus exp_B (5 bit)

Simple explanation: Computes exp_A − exp_B. This gives a positive shift amount when A has the larger exponent. Like measuring how far apart two numbers are on a number line.

Subtractor for exp_B minus exp_A (5 bit)

Simple explanation: Computes exp_B − exp_A. This gives a positive shift amount when B has the larger exponent.

We need both subtractors because subtraction of larger minus smaller always gives a positive result. We cannot know in advance which is larger so we compute both and then pick the correct one.

2-to-1 MUX for larger_exp selection (5 bit)

Simple explanation: Picks which exponent to pass forward as the larger one. When swap_flag=0 (A is larger) it outputs exp_A. When swap_flag=1 (B is larger) it outputs exp_B. The larger exponent becomes the reference exponent for the result.

2-to-1 MUX for shift_amount selection (5 bit)

Simple explanation: Picks the correct (always positive) shift amount. When swap_flag=0 it outputs exp_A minus exp_B. When swap_flag=1 it outputs exp_B minus exp_A. This ensures shift_amount is always a positive number representing how many positions to shift the smaller mantissa.

SUB6 — Barrel Shifter

What This Subcircuit Does

This subcircuit right-shifts the smaller mantissa by the shift amount from SUB5. This aligns the decimal points of both numbers before addition. It is like moving digits to the right when you align numbers for hand addition.

For example if A has exponent 17 and B has exponent 15 then B's mantissa needs to shift right by 2 positions to align with A.

It also handles the overflow case: if the shift amount is 11 or more the entire mantissa shifts out and becomes zero.

Components and What Each One Does

5-to-2 Splitter

Simple explanation: The shift amount is 5 bits wide but the Logisim Shifter component only accepts a 4-bit shift distance for 11-bit data. This splitter separates the 5-bit shift amount into:

- Bits 0–3 (lower 4 bits) → goes to the Shifter as the shift distance

- Bit 4 (the top bit) → if this is 1 the shift is 16 or more which means result is definitely zero

Comparator (4 bit, unsigned)

Simple explanation: Compares the lower 4 bits of the shift amount against the constant value 10. If the lower 4 bits are greater than 10 then the shift amount is 11 or more meaning the mantissa shifts completely out and the result is zero.

Constant 10 (4 bit, value = 10)

Simple explanation: A hardwired value of 10 used as the comparison reference. Any shift of more than 10 positions on an 11-bit mantissa gives zero.

OR Gate for force_zero (1 bit, 2 inputs)

Simple explanation: Combines two conditions that both indicate the shift is too large:

- Lower 4 bits of shift > 10

- Bit 4 of shift = 1 (shift >= 16)

If either condition is true the output must be forced to zero.

Shifter (11 bit, Logical Right)

Simple explanation: This is the main shifting component. It takes the 11-bit mantissa and shifts it right by the specified number of positions. Each right shift is like dividing by 2. Empty positions on the left are filled with 0. A logical right shift means zeros are inserted from the left — this is correct for mantissa alignment.

Constant 0 (11 bit)

Simple explanation: A hardwired 11-bit zero value. Used as the output when force_zero is active — when the shift amount is too large the result is all zeros.

2-to-1 MUX for output selection (11 bit)

Simple explanation: The final selector. When force_zero=0 it outputs the Shifter result. When force_zero=1 it outputs the zero constant. This ensures the output is always correct regardless of shift amount.

SUB7 — Effective Operation Decider

What This Subcircuit Does

This subcircuit looks at the sign of A and the effective sign of B (after SUB3 flipped it) and decides whether the mantissa operation should be addition or subtraction.

The rule is: same signs → add the magnitudes. Different signs → subtract the magnitudes.

For example:

- (+2.5) + (+1.5) → same signs → ADD mantissas

- (+2.5) + (−1.5) → different signs → SUBTRACT mantissas

- (−2.5) + (−1.5) → same signs → ADD mantissas

Components and What Each One Does

XNOR Gate (1 bit, 2 inputs)

Simple explanation: XNOR outputs 1 when both inputs are THE SAME and 0 when they are DIFFERENT. This is exactly what we need:

- Same signs (both 0 or both 1) → XNOR outputs 1 → ADD

- Different signs (one 0, one 1) → XNOR outputs 0 → SUBTRACT

Think of it as a compatibility checker. When sign_A and eff_sign_B are compatible (same) we add. When they are incompatible (different) we subtract.

SUB8 — Mantissa Adder/Subtractor

What This Subcircuit Does

This is the arithmetic heart of the circuit. It takes two aligned 11-bit mantissas and either adds or subtracts them based on eff_op from SUB7.

It uses a clever technique called 2's complement to perform subtraction using only an adder:

- A − B = A + (~B) + 1

- ~B means flip all bits of B

- The +1 comes from the carry-in

It outputs a 12-bit result (one extra bit for carry overflow) and a result_negative flag indicating whether the subtraction result was negative.

Components and What Each One Does

NOT Gate for inverting B (11 bit)

Simple explanation: Flips all 11 bits of shifted_mant_B to create ~B. This is the first step of 2's complement subtraction. Like turning every 0 into 1 and every 1 into 0 in the binary number.

2-to-1 MUX for B operand selection (11 bit)

Simple explanation: Chooses which version of B to use in the Adder:

- When eff_op=1 (adding) → use B unchanged

- When eff_op=0 (subtracting) → use ~B (inverted B)

This single MUX lets us use one Adder for both addition and subtraction.

NOT Gate for carry_in (1 bit)

Simple explanation: Generates the carry_in signal for the Adder. When subtracting (eff_op=0) we need carry_in=1 to complete the 2's complement (adding ~B + 1). When adding (eff_op=1) we need carry_in=0. NOT(eff_op) gives us exactly this.

Adder — main_adder (11 bit)

Simple explanation: This is the main addition unit. It adds three things:

- mant_A (11 bits)

- B_operand (11 bits — either B or ~B)

- carry_in (1 bit — either 0 or 1)

It outputs an 11-bit sum and a 1-bit carry_out. When the sum exceeds 11 bits the carry_out becomes 1 indicating overflow.

AND Gate for MSB (1 bit, 2 inputs)

Simple explanation: Computes: msb = carry_out AND eff_op

This gives us bit 11 (the most significant bit) of the 12-bit result. We only use carry_out as a real overflow bit during addition. During subtraction carry_out has a different meaning (it tells us the sign) so we mask it out with eff_op.

West-facing Splitter joiner for 12-bit raw result

Simple explanation: Joins the 11-bit sum (bits 0–10) and the 1-bit MSB (bit 11) into a single 12-bit value. The MSB becomes the most significant bit of the result. This is how we capture addition overflow in the extra 12th bit.

NOT Gate for NOT(carry_out) (1 bit)

Simple explanation: Inverts the carry_out. During subtraction carry_out=0 means the result is negative (A < B). Inverting it gives us a signal that is 1 when the result is negative.

AND Gate for result_negative (1 bit, 2 inputs)

Simple explanation: Combines NOT(eff_op) and NOT(carry_out):

- NOT(eff_op) = 1 only during subtraction

- NOT(carry_out) = 1 only when A < B

Both must be true for the result to be negative. result_negative = 1 means the subtraction gave a negative answer.

NOT Gate for inverting sum (11 bit)

Simple explanation: First step of computing the magnitude when result is negative. Flips all 11 bits of the sum. Used only when result_negative=1.

Adder — neg_adder (11 bit)

Simple explanation: Second step of computing magnitude. Adds 1 to the inverted sum (completing the 2's complement). This gives the absolute value of the negative result. For example if the raw sum was −512, the magnitude is 512.

Constant 1 for neg_adder carry-in (1 bit)

Simple explanation: Hardwired 1 always feeding into neg_adder's carry-in. This provides the +1 needed to complete the 2's complement negation.

Constant 0 for neg_adder bottom input (11 bit)

Simple explanation: The Adder component needs two data inputs. Since we only want to add 1 (via carry_in) we feed 0 into the second data input. Adding 0 + inverted_sum + 1 gives us the magnitude.

Bit Extender zero_extend (11 bit to 12 bit, Zero type)

Simple explanation: The magnitude from neg_adder is 11 bits but the final output needs to be 12 bits. This component adds a 0 at the most significant bit position to make it 12 bits. It is like adding a leading zero to a number — it does not change the value but makes the width match.

2-to-1 MUX — final_mux (12 bit)

Simple explanation: The final selector that decides which 12-bit value to output:

- When result_negative=0 → output the raw 12-bit result (positive answer)

- When result_negative=1 → output the zero-extended magnitude (absolute value of negative answer)

The sign information is handled separately by SUB10. This subcircuit only outputs the magnitude.

SUB9 — Normaliser

What This Subcircuit Does

After addition or subtraction the result mantissa may not be in proper IEEE 754 normalised form. This subcircuit fixes that.

A normalised IEEE 754 number has exactly one 1 before the decimal point (like 1.something × 2^power). After arithmetic the result might have:

- Two digits before the decimal (overflow) → shift right by 1, increase exponent by 1

- Zeros before the first 1 (leading zeros) → shift left, decrease exponent accordingly

This subcircuit also detects if the result overflowed to infinity or is zero.

Components and What Each One Does

Comparator for zero detection (12 bit, unsigned)

Simple explanation: Compares the 12-bit result against zero. If result = 0 then result_is_zero=1. This catches the case where two equal numbers were subtracted giving zero.

Constant 0 for zero comparison (12 bit)

Simple explanation: Hardwired 12-bit zero used as the comparison reference for the zero detector comparator.

12-to-2 Splitter — split_top

Simple explanation: Separates the 12-bit result into:

- Bits 0–10 (11 bits) → the main mantissa body

- Bit 11 (1 bit) → the carry overflow flag

Bit 11 being 1 means addition overflowed into the 12th bit and we need to right shift.

11-to-11 Splitter — split_11bits

Simple explanation: Takes the 11-bit lower portion and separates each bit onto its own wire. This is needed so the Priority Encoder can check each bit individually to find the highest set bit.

Priority Encoder (Select Bits=4)

Simple explanation: This component finds the position of the highest set bit in the 11-bit result. Think of it as asking "which is the leftmost lit lamp in a row of 11 lamps?"

If the highest set bit is at position 10 (bit10=1) the encoder outputs 10. If it is at position 8 (bit8=1) the encoder outputs 8. This tells us where the leading 1 is so we know how many positions to shift left to normalise.

We connect bit10 to in10 (highest priority) and bit0 to in0 (lowest priority). Unused inputs in11–in15 connect to Constant 0.

Bit Extender — extend_enc (4 bit to 5 bit, Zero type)

Simple explanation: The Priority Encoder outputs 4 bits but we need 5 bits to match the 5-bit subtractor that calculates shift amount. This extender adds a 0 at the top to make it 5 bits wide without changing the value.

Constant 10 — const_10 (4 bit, value = 10)

Simple explanation: Hardwired value 10 (decimal). The leading 1 of a normalised 11-bit mantissa should be at position 10. So the shift amount needed = 10 minus (current highest bit position).

Subtractor — shift_calc (4 bit)

Simple explanation: Computes: shift_amount = 10 − encoder_output

For example:

- If highest bit is at position 10 → shift = 10−10 = 0 (already normalised)

- If highest bit is at position 8 → shift = 10−8 = 2 (need to shift left by 2)

- If highest bit is at position 7 → shift = 10−7 = 3 (need to shift left by 3)

Left Shifter (12 bit, Logical Left)

Simple explanation: Shifts the 12-bit result left by the calculated shift amount. Each left shift moves all bits one position to the left and fills the rightmost position with 0. This is like multiplying by 2 for each shift position. Used to normalise results that have leading zeros.

Right Shifter (12 bit, Logical Right)

Simple explanation: Shifts the 12-bit result right by exactly 1 position. Used only when bit11=1 (carry overflow from addition). Moving right by 1 fixes the overflow and the exponent gets incremented by 1 to compensate.

Constant 1 for right shifter distance (4 bit)

Simple explanation: Hardwired 4-bit value of 1. The right shifter always shifts by exactly 1 position (only needed when carry overflow occurred).

2-to-1 MUX — shift_mux (12 bit)

Simple explanation: Selects which shifted result to use:

- When bit11=0 → use left shifted result (normal normalisation)

- When bit11=1 → use right shifted result (carry overflow case)

12-to-2 Splitter — strip_hidden

Simple explanation: After shifting the normalised mantissa has its leading 1 at bit position 10. IEEE 754 does not store this leading 1 (it is the hidden bit). This splitter removes it by taking only bits 0–9 (10 bits) as the final mantissa and discarding bits 10–11.

Subtractor — exp_dec (5 bit)

Simple explanation: Adjusts the exponent downward when we shifted left. Computes: final_exp = larger_exp − shift_amount. Each left shift of the mantissa means we divided by 2 each time so we must decrease the exponent to compensate.

Adder — exp_inc (5 bit)

Simple explanation: Adjusts the exponent upward by 1 when carry overflow occurred (right shift case). Computes: final_exp = larger_exp + 1. Each right shift of the mantissa means we multiplied by 2 so we increase the exponent by 1.

Constant 1 for exp_inc (5 bit)

Simple explanation: Hardwired 5-bit value of 1 fed into exp_inc adder. The exponent only ever increments by exactly 1 in the overflow case.

2-to-1 MUX — exp_mux (5 bit)

Simple explanation: Selects the correct final exponent:

- When bit11=0 → use exp_dec output (left shift case)

- When bit11=1 → use exp_inc output (right shift/overflow case)

Comparator — overflow_check (5 bit, unsigned)

Simple explanation: Checks if the final exponent exceeds 30. In IEEE 754 half-precision the maximum valid exponent is 30 (biased). If final_exp > 30 the number is too large to represent and the result is infinity.

Constant 30 — const_30 (5 bit, value = 30)

Simple explanation: Hardwired 5-bit value of 30 (hex 1E) used as the comparison reference for overflow detection. If final_exp is greater than this value the result overflows to infinity.

SUB10 — Sign Resolver (Final Corrected Version)

What This Subcircuit Does

This subcircuit determines the final sign bit of the result. It considers four factors:

- sign_A — sign of input A

- eff_sign_B — effective sign of B after SUB flipping

- eff_op — were we adding or subtracting mantissas?

- swap_flag — did B have larger exponent?

- result_negative — was the subtraction result negative (B's mantissa was larger)?

- result_is_zero — is the result zero (always output positive zero)?

The key insight is:

- During addition → sign_A determines the result sign (both inputs had same sign)

- During subtraction where A >= B → sign_A determines result sign (A dominated)

- During subtraction where B > A → eff_sign_B determines result sign (B dominated)

B > A happens when swap_flag=1 (B had larger exponent) OR result_negative=1 (same exponent but B's mantissa was larger).

Components and What Each One Does

OR Gate (1 bit, 2 inputs)

Simple explanation: Combines swap_flag and result_negative into a single "B dominated" signal. If either is true it means B was the larger number and its sign should dominate the result. Think of it as asking "was B bigger in ANY way — either by exponent or by mantissa?"

NOT Gate — not_eff_op (1 bit)

Simple explanation: Inverts eff_op. When eff_op=1 (addition) NOT gives 0. When eff_op=0 (subtraction) NOT gives 1. This is used to ensure the sign logic only activates for subtraction operations. During addition sign_A always wins regardless.

AND Gate — use_B_sign_gate (1 bit, 2 inputs)

Simple explanation: Combines NOT(eff_op) and (swap_flag OR result_negative):

- use_B_sign = NOT(eff_op) AND (swap_flag OR result_negative)

This is 1 only when BOTH conditions are true:

- We are subtracting (NOT(eff_op)=1)

- B was the larger number (OR gate=1)

When use_B_sign=1 we use eff_sign_B as the result sign. When use_B_sign=0 we use sign_A.

2-to-1 MUX — sign_mux (1 bit)

Simple explanation: Makes the actual sign selection:

- When use_B_sign=0 → output sign_A

- When use_B_sign=1 → output eff_sign_B

This gives us the raw sign before the zero check.

NOT Gate — not_zero (1 bit)

Simple explanation: Inverts result_is_zero. When result is zero this NOT gives 0. This 0 will force the final sign to 0 through the AND gate below — ensuring zero always has positive sign per IEEE 754 rules.

AND Gate — final_and (1 bit, 2 inputs)

Simple explanation: The final sign gate. Computes: final_sign = raw_sign AND NOT(result_is_zero)

When result_is_zero=1 → NOT gives 0 → AND forces output to 0 → always positive zero. When result_is_zero=0 → NOT gives 1 → AND passes through raw_sign unchanged.

SUB11 — Output Packer with Special Case Merge

What This Subcircuit Does

This is the last subcircuit. It assembles the final 16-bit IEEE 754 result from all the pieces computed by earlier subcircuits. It handles two paths:

Special path: If is_special=1 (caught by SUB1 and SUB2) simply output special_result directly.

Normal path: Assemble the 16-bit result from:

- final_sign (1 bit) at bit position 15

- final_exp (5 bits) at bit positions 14–10

- norm_mantissa (10 bits) at bit positions 9–0

It also overrides the normal packed result with infinity or zero patterns when result_is_inf or result_is_zero is detected by SUB9.

Components and What Each One Does

West-facing Splitter joiner — join_normal (16 bit, Fan Out=3)

Simple explanation: Takes three separate fields and joins them into one 16-bit result:

- Top-left port (1 bit) receives final_sign → goes to bit 15

- Middle-left port (5 bits) receives final_exp → goes to bits 14–10

- Bottom-left port (10 bits) receives norm_mantissa → goes to bits 9–0

The wide right port outputs the complete packed 16-bit normal result. Like putting three puzzle pieces together into one picture.

West-facing Splitter joiner — join_inf (16 bit, Fan Out=3)

Simple explanation: Same structure as join_normal but builds the infinity representation:

- Top-left port receives final_sign (so we get correct + or − infinity)

- Middle-left port receives const_inf_exp (11111) — always infinity exponent

- Bottom-left port receives const_zero_mant (0000000000) — infinity has zero mantissa

Constant — const_inf_exp (5 bit, value = 11111)

Simple explanation: Hardwired 5-bit value 31 (all ones = 11111). In IEEE 754 half-precision exponent = 11111 with mantissa = 0 means infinity. This constant provides that infinity exponent.

Constant — const_zero_mant (10 bit, value = 0)

Simple explanation: Hardwired 10-bit zero. Infinity and zero results both need a mantissa of all zeros. This constant provides that.

Constant — const_zero_16b (16 bit, value = 0)

Simple explanation: Hardwired complete 16-bit zero representing positive zero (+0.0). Used when result_is_zero=1 to output the zero representation directly.

2-to-1 MUX — mux_inf (16 bit)

Simple explanation: First selector in the cascade. Chooses between normal result and infinity result:

- When result_is_inf=0 → output normal_packed (from join_normal)

- When result_is_inf=1 → output inf_packed (from join_inf)

2-to-1 MUX — mux_zero (16 bit)

Simple explanation: Second selector. Chooses between previous result and zero:

- When result_is_zero=0 → pass through mux_inf output

- When result_is_zero=1 → output const_zero_16b (all zeros)

This overrides anything that came before with a clean zero when the result is zero.

2-to-1 MUX — final_mux (16 bit)

Simple explanation: The ultimate final selector. This is where the special case path and the normal arithmetic path merge:

- When is_special=0 → output mux_zero result (normal computed answer)

- When is_special=1 → output special_result from SUB2 (NaN, infinity, or zero from special case detection)

is_special=1 completely overrides everything computed by the arithmetic pipeline and directly outputs the answer that SUB2 pre-computed.

TOP — Main Circuit

What This Circuit Does

The main circuit is the complete IEEE 754 Half-Precision Floating Point Adder/Subtractor. It connects all 11 subcircuits together in the correct order and also contains two helper MUX components.

It takes three inputs from the outside world:

- A (16 bits) — first IEEE 754 number

- B (16 bits) — second IEEE 754 number

- SUB (1 bit) — 0 for addition, 1 for subtraction

It produces one output:

- Result (16 bits) — the IEEE 754 answer

Components and What Each One Does

All 11 Subcircuits

Each subcircuit appears as a rectangular block with labeled ports. They are connected in the order described throughout this document.

Helper MUX — mant_swap_mux (11 bit)

Simple explanation: When the exponents are different we need to shift the SMALLER mantissa. swap_flag from SUB5 tells us which number had the smaller exponent:

- swap_flag=0 (A is larger) → we need to shift B's mantissa → input 0 = mant_B

- swap_flag=1 (B is larger) → we need to shift A's mantissa → input 1 = mant_A

Output goes to SUB6 for shifting.

Helper MUX — mant_larger_mux (11 bit)

Simple explanation: The LARGER mantissa goes to SUB8 unshifted as the reference value:

- swap_flag=0 (A is larger) → input 0 = mant_A → goes to SUB8 mant_A

- swap_flag=1 (B is larger) → input 1 = mant_B → goes to SUB8 mant_A

Both helper MUXes use the same swap_flag from SUB5 as their select signal. Together they ensure the correct mantissas go to the correct places regardless of which input had the larger exponent.